This version of Planning Advisory Note 65: New planning provisions for apartment developments – Amendment VC136 has been prepared for use with screen reader software. The printed publication contains various design features that have been necessarily omitted from this version. In other respects this document contains identical text to that in the PDF version of the document which is available at www.planning.vic.gov.au

## Advisory Note 66

New planning provisions for apartment developments – Amendment VC136

### April 2017

This advisory note provides information about the changes made to the Victoria Planning Provisions and all planning schemes by Amendment VC136 to introduce new standards for apartment developments in Victoria.

# How does Amendment VC136 change the Victoria Planning Provisions (VPP) and all planning schemes?

Amendment VC136 amends the VPP and all planning schemes by:

- Amending Clause 55 (Two or more dwellings on a lot and residential buildings) to introduce the new apartment standards.

- Introducing a new Clause 58 (Apartment developments) into the Particular Provisions.

- Deleting Clause 52.35 (Urban context report and design response) for residential development of five or more storeys.

- Amending the following zones to reference the new apartment standards (Clause 32.04 (Mixed Use Zone), Clause 32.05 (Township Zone), Clause 32.07 (Residential Growth Zone), Clause 32.08 (General Residential Zone), Clause 32.09 (Neighbourhood Residential Zone), Clause 34.01 (Commercial 1 Zone), Clause 37.01 (Special Use Zone), Clause 37.02 (Comprehensive Development Zone), Clause 37.04 (Capital City Zone), Clause 37.05 (Docklands Zone), Clause 37.06 (Priority Development Zone) and Clause 37.08 (Activity Centre Zone) ).

- Amending Clause 43.05 (Neighbourhood  Character Overlay) to prevent variation of the new apartment standards.

- Amending Clause 72 (General Terms) to introduce a definition for the term ‘Apartment’.

# What is an apartment?

The VPP and all planning schemes have been updated to include a definition of Apartment under Clause 72 (General terms):

A dwelling located above the ceiling level or below the floor level of another dwelling and is part of a building containing two or more dwellings.

# What are the new apartment standards?

There are 16 new apartment standards:

- Building setback

- Functional layout

- Room depth

- Windows

- Storage

- Noise impacts

- Energy efficiency

- Natural ventilation

- Private open space

- Communal open space

- Solar access to communal outdoor open space

- Landscaping

- Accessibility

- Building entry and circulation

- Waste and recycling

- Integrated water and stormwater management

# What type of development do the new apartment standards apply to?

The new apartment standards apply to all apartment developments in Victoria, except for the Building setback standard.

The Building setback standard does not apply to apartment developments of four storeys or less in a residential zone.  The requirements of Clause 55.04-1 (Side and rear setback) continue to apply to these apartment developments.

# Do the new apartment standards apply to a residential building?

No, the new apartment standards do not apply to a residential building as defined in the VPP.

A residential building of four storeys or less in a residential zone will continue to be assessed under the provisions of Clause 55 (Two or more dwellings on a lot and residential buildings).

A residential building of five or more storeys will continue to be assessed against relevant policies, zones and overlays.

# How do the new apartment provisions operate?

The new apartment provisions operate in the same way that Clauses 54 and 55 operate to assess residential development in the VPP.

The new apartment provisions contain Objectives, Standards and Decision guidelines.

Objectives describe the desired outcome to be achieved in the completed development.

A Standard contains the requirements to meet the objective. A standard should normally be met. However, if the responsible authority is satisfied that an alternative design solution meets the objective, the alternative design solution may be considered.

Decision guidelines set out the matters that the responsible authority must consider before deciding if an application meets the objectives. When an alternative design solution is proposed, the effect of the design solution on the achievement of other objectives should be considered.

# Can the new apartment standards be varied by zones and overlays?

The new apartment standards contained in Clause 55.07 cannot be varied in a schedule to the zone. A select number of the existing standards under Clause 55 continue to have the ability to be varied by schedules to the zones.

The Neighbourhood Character Overlay will continue to allow variations to most of the standards of Clause 55 except for the new apartment standards in Clause 55.07 and existing key amenity standards such as overshadowing and daylight to new and existing windows.

Apartment standards in Clause 58 cannot be varied in a schedule to the zone or an overlay, except for the Building setback standard (Clause 58.04-1).

# How does Amendment VC136 affect existing planning permit applications?

Clause 58 and the amended Clause 55 do not apply to applications lodged before the introduction of Amendment VC136 on 13 April 2017.

The transitional arrangements also apply to an application lodged for an amendment of a permit where the original permit application was lodged before the introduction of Amendment VC136 on 13 April 2017.

# How are the new apartment standards given effect in Clause 55 and Clause 58?

Clause 55 has been amended to include the new apartment standards at Clause 55.07.

Apartment developments of four storeys or less (excluding a basement) in a General Residential Zone, Residential Growth Zone, Mixed Use Zone, Township Zone and Neighbourhood Residential Zone will continue to be assessed against the amended Clause 55.

Clause 58 includes relevant standards from Clause 55 as well as all of the new apartment standards.

Apartment developments of five or more storeys, excluding a basement in a General Residential Zone, Residential Growth Zone, Mixed Use Zone and Township Zone and all apartment developments in a Commercial 1 Zone, Special Use Zone, Comprehensive Development Zone, Capital City Zone, Docklands Zone, Priority Development Zone and Activity Centre Zone are assessed against Clause 58.

Table 1 shows the new and existing standards across both clauses.

## Apartment standards under Clause 55 and Clause 58

### CLAUSE 55 TWO OR MORE DWELLINGS ON A LOT AND RESIDENTIAL BUILDINGS

### Apartment development of four storeys or less in a residential zone

### Neighbourhood and site description and design response

EXISTING STANDARDS

- B1: Neighbourhood character

- B2: Residential policy

- B3: Dwelling diversity

- B4: Infrastructure

- B5: Integration with street

- B6: Street setback

- B7: Building height

- B8: Site coverage

- B9: Permeability

- B10: Energy efficiency – N/A. Refer to B35

- B11: Open space – N/A. Refer to B36

- B12: Safety

- B13: Landscaping

- B14: Access

- B15: Parking location

- B17: Side and rear setback

- B18: Walls on boundaries

- B19: Daylight to existing windows

- B20: North facing windows

- B21: Overshadowing open space

- B22: Overlooking

- B23: Internal views

- B24: Noise impacts – N/A. Refer to B40

- B25: Accessibility – N/A. Refer to B41

- B26: Dwelling entry – N/A. Refer to B42

- B27: Daylight to new windows

- B28: Private open space

- B29: Solar access to open space

- B30: Storage – N/A. Refer to B44

- B31: Design detail

- B32: Front fences

- B33: Common property

- B34: Site services

Clause 55.07 – Apartment developments

- B35: Energy efficiency

- B36: Communal open space

- B37: Solar access to communal outdoor open space

- B38: Deep soil areas and canopy trees

- B39: Integrated water and stormwater management

- B40: Noise impacts

- B41: Accessibility

- B42: Building entry and circulation

- B43: Private open space above ground floor

- B44: Storage

- B45: Waste and recycling

- B46: Functional layout

- B47: Room depth

- B48: Windows

- B49: Natural ventilation

### CLAUSE 58 APARTMENT DEVELOPMENTS

### Apartment development of five or more storeys in a residential zone, plus All apartment developments in other zones

### Urban context report and design response

NEW STANDARDS

- D1: Urban context

- D2: Residential policy

- D3: Dwelling diversity

- D4: Infrastructure

- D5: Integration with street

- D6: Energy efficiency

- D7: Communal open space

- D8: Solar access to communal outdoor open space

- D9: Safety

- D10: Landscaping

- D11: Access

- D12: Parking location

- D13: Integrated water and stormwater management

- D14: Building setback

- D15: Internal views

- D16: Noise impacts

- D17: Accessibility

- D18: Building entry and circulation

- D19: Private open space

- D20: Storage

- D21: Common property

- D22: Site services

- D23: Waste and recycling

- D24: Functional layout

- D25: Room depth

- D26: Windows

- D27: Natural ventilation

© The State of Victoria Department of Environment, Land, Water & Planning 2017

This work is licensed under a Creative Commons Attribution 4.0 International licence. You are free to re-use the work under that licence, on the condition that you credit the State of Victoria as author. The licence does not apply to any images, photographs or branding, including the Victorian Coat of Arms, the Victorian Government logo and the Department of Environment, Land, Water and Planning (DELWP) logo. To view a copy of this licence, visit http://creativecommons.org/licenses/by/4.0/

Disclaimer

This publication may be of assistance to you but the State of Victoria and its employees do not guarantee that the publication is without flaw of any kind or is wholly appropriate for your particular purposes and therefore disclaims all liability for any error, loss or other consequence which may arise from you relying on any information in this publication.

Accessibility

If you would like to receive this publication in an alternative format, please telephone the DELWP Customer Service Centre on 136186, email customer.service@delwp.vic.gov.au (or relevant address), or via the National Relay Service on 133 677 www.relayservice.com.au This document is also available on the internet at www.planning.vic.gov.au

DOCUMENT ENDS