This version of Advisory Note 33 Amendment VC83 Bushfire protection – Community fire refuge and private bushfire shelter exemptions has been prepared for use with screen reader software. The printed publication may contain photographs, captions and design features that have been necessarily omitted from this version. In other respects this document contains identical text to that in the PDF version of the document which is available at www.dpcd.vic.gov.au/planning .

## Advisory Note 33

# Bushfire protection – Community fire refuge and private bushfire shelter exemptions

## Amendment VC83

### November 2011

This advisory note provides detail about the new planning permit exemptions for buildings and works associated with a community fire refuge and a private bushfire shelter.

## Introduction

Amendment VC83 introduced clause 52.48 Bushfire protection: exemptions. The new exemptions relating to community fire refuges and private bushfire shelters can be found at sub clauses 52.48-3 and 52.48-4 of the Victoria Planning Provisions and all planning schemes.

## Private bushfire shelter

### What does the new exemption do?

Clause 52.48-4 exempts any requirement in the planning scheme relating to the construction of a building or the construction or carrying out of works associated with a private bushfire shelter (a Class 10c building within the meaning of the Building Regulations 2006), provided the total area of all buildings and works does not exceed 30 square metres.

### Does the exemption apply everywhere?

No. The exemption does not apply to land in the Urban Floodway Zone, Erosion Management Overlay, Floodway Overlay, Land Subject to Inundation Overlay, Special Building Overlay or Heritage Overlay. In these areas normal planning permit requirements continue to apply.

To find out what zone land is in or if an overlay applies to a property:

- obtain a Planning Property Report at www.land.vic.gov.au

- download the PlanningVIC: Planning Property Report app for iPad and iPhone via www.dpcd.vic.gov.au/planning

- use the online mapping tool Planning Maps Online at www.land.vic.gov.au

- view the relevant planning scheme maps at www.dpcd.vic.gov.au/planning/planningschemes

- contact your local council.

The planning permit exemption also does not apply to land in the Shire of Yarra Ranges. For more information about the planning requirements for private bushfire shelters in the Shire of Yarra Ranges contact the council on 1300 368 333.

### Does the exemption apply to vegetation removal?

No. The exemption does not apply to the removal, destruction or lopping of vegetation. If vegetation is required to be removed to enable buildings and works associated with a private bushfire shelter a planning permit may be required. However, existing planning permit exemptions for vegetation removal continue to apply.

### What is a private bushfire shelter?

Regulation 115 of the Building Regulations 2006 defines a private bushfire shelter as a building that may, as a last resort, provide shelter for occupants from the immediate life threatening effects of a bushfire event.

A building permit must be obtained for a Class 10c building, a building being a private bushfire shelter. Buildings and works must not commence until a building permit has been issued for a private bushfire shelter. A building permit can only be issued if the private bushfire shelter is in association with a Class 1 building.

A private bushfire shelter must comply with the performance requirements set out in the Building Regulations including safe access and egress and a tenable environment within the shelter for the period of occupancy.

The performance requirements are met if the private bushfire shelter is designed and constructed in accordance with the Performance Standard for Private Bushfire Shelters 2010 (Part 1), Australian Building Codes Board.

To obtain a building permit for a private bushfire shelter a landowner must either:

- purchase a shelter which has been accredited by the Building Regulations Advisory Committee as meeting the performance requirements in the regulations; or

- apply to the Building Appeals Board for a determination that the non-accredited bushfire shelter complies with the regulations; or

- obtain certification from a fire safety engineer who did not design the shelter to satisfy a building surveyor that a non-accredited bushfire shelter meets the requirements of the regulations; or

- rely on a Certificate of Conformity issued by the Australian Building Codes Board which states that the alternative solution complies with that performance requirement.

More information about the regulation and accreditation of private bushfire shelters is available from the Building Commission website at www.buildingcommission.com.au.

## Community fire refuge

### What does the new exemption do?

Clause 52.48-3 exempts any requirement in the planning scheme relating to the construction of a building or the construction or carrying out of works associated with a community fire refuge in accordance with the Building Regulations 2006.

### Where does the exemption apply?

The planning permit exemption applies to all land, except land in the Shire of Yarra Ranges. For more information about the planning requirements for community fire refuges in the Shire of Yarra Ranges contact the council on 1300 368 333.

### Does the exemption apply to vegetation removal?

No. The exemption does not apply to the removal, destruction or lopping of vegetation. If vegetation is required to be removed to enable buildings and works associated with a community fire refuge a planning permit may be required. However, existing planning permit exemptions for vegetation removal continue to apply.

### What is a community fire refuge?

Regulation 115A of the Building Regulations 2006 defines a community fire refuge as a building, or part of a building, to be used by the public for short-term shelter from a fire front during a bushfire event.

A building permit must be obtained for a Class 9d building, a building being a community fire refuge. Buildings and works must not commence until a building permit has been issued for a community fire refuge.

A community fire refuge must comply with the performance requirements set out in the Building Regulations including safe access and egress and a tenable environment within the shelter for the period of occupancy.

## More information

More information about the new bushfire provisions introduced by Amendment VC83 is provided in:

Advisory note 39 – Bushfire protection: Vegetation exemption

Advisory note 40 – Bushfire protection: Bushfire planning provisions

Practice note 64 – Local planning for bushfire protection

Practice note 65 – Bushfire Management Overlay and bushfire protection: planning requirements

More bushfire related information is available at 
www.dpcd.vic.gov.au/planning/bushfire

----------------------------------------------------

This advisory note provides information about the bushfire planning provisions introduced into the Victoria Planning Provisions and planning schemes by Amendment VC83.

## What are the bushfire provisions?

The bushfire planning provisions include:

- The State Planning Policy Framework (SPPF) (clause 13.05 – Bushfire)

- The Local Planning Policy Framework (Municipal Strategic Statement (MSS) and local planning policy where relevant)

- The Bushfire Management Overlay (BMO) (clause 44.06)

- Bushfire protection: planning requirements (clause 52.47)

- Bushfire protection: exemptions (clause 52.48), native vegetation (clauses 52.16 and 52.17) and overlays which seek to manage vegetation

- Decision guidelines (clause 65)

- Referrals (clause 66)

- Definitions (clause 72).

## Ministerial Direction 11: Strategic Assessment of Amendments

Ministerial Direction 11 and its associated practice note requires planning authorities to evaluate how an amendment has addressed bushfire risk. If relevant, a discussion on bushfire matters must be included in the explanatory report.

## SPPF

The SPPF provides the broad framework for integrating bushfire policy and provisions into the planning scheme. Its objective is to assist in strengthening community resilience to bushfire. The SPPF provides guidance on how bushfire considerations are to be treated in planning schemes and in planning decisions. It:

- provides a framework for hazard identification and risk assessments in the planning system

- ensures that bushfire considerations are taken into account in strategic and settlement planning

- provides direction to planning authorities for implementing bushfire matters in a planning scheme

- provides development control strategies for areas affected by the bushfire hazard, such as the BMO.

## Local Planning Policy Framework (LPPF)

In the LPPF, the MSS establishes the strategic framework for the municipality and should show how it supports and implements the SPPF. The SPPF and MSS together provide the strategic basis for considering bushfire through planning schemes. If necessary, a council may also use a local planning policy for bushfire to guide decision making in areas affected by a bushfire hazard.

See Practice note 64: Local planning for bushfire protection.

### Bushfire Management Overlay (BMO)

The BMO is used to guide the development of land in areas of high bushfire hazard. It requires consideration of the location, design and construction of development and the implementation of bushfire protection measures. The BMO may be supported by an associated schedule. The ‘parent’ provisions in the BMO provide scope to use a schedule to tailor the BMO to specific local circumstances. A schedule to the BMO is prepared by the council. Where no schedule is in place, the statewide provisions of the BMO apply.

See Practice note 65: Bushfire Management Overlay and bushfire protection: planning requirements.

## How are BMO areas identified?

BMO areas are shown on planning scheme maps as either BMO or WMO.

Areas shown on planning scheme maps as WMO were introduced into planning schemes before 
18 November 2011. The WMO is applied to forests greater than five hectares in size and with a vegetation density of greater than 80%. A forest of this size and density can generate fuel loads capable of supporting extreme and unpredictable bushfire behaviour, for example a crown fire (a fire in the treetops). A crown fire produces extreme radiant heat, flame lengths and embers.

Areas shown on planning scheme maps as BMO have been introduced into planning schemes since 18 November 2011. The BMO is based on the bushfire hazard and is applied to areas of extreme fuel loads where there is a potential for extreme bushfire behaviour such as a crown fire and extreme ember attack and radiant heat. The BMO takes into account vegetation, weather characteristics and slope.

## Introducing new BMO mapping

Initially, the area to which the BMO will apply will be the same as that to which the WMO applied.

The Department of Planning and Community Development will work with councils and the Country Fire Authority to progressively introduce new BMO mapping into planning schemes. Over time this will replace WMO mapping so that the maps are based on up-to-date hazard information, verified with local knowledge.

## Bushfire protection: planning requirements (clause 52.47)

Specific objectives, standards and decision guidelines are set out in clause 52.47 of all planning schemes and these inform decisions and requirements under the BMO. The matters to be considered include:

- location, layout and siting

- defendable space and construction

- water supply and access

- implementation of bushfire protection measures.

## Bushfire protection: exemptions (clause 52.48), native vegetation (clauses 52.16 and 52.17) and overlays which manage vegetation

Permit exemptions for bushfire protection are set out in clauses 52.48, 52.16 and 52.17, and in environmental overlays which seek to manage vegetation. These provisions set out permit exemptions for:

- creating defendable space

- removing vegetation along a fence line

- roadside vegetation management by a road manager

- a community fire refuge and a private bushfire shelter.

For further detail see More information on page 3.

## Decision guidelines (clause 65)

Whether or not the site is affected by the BMO, clause 65 of the planning scheme extends the consideration of bushfire issues to all planning permit applications. Clause 65 requires that for the approval of an application or plan, the council must consider, among other things, the degree of bushfire hazard associated with the location of the land and the use, development or management of the land so as to minimise bushfire risk.

## Referrals (clause 66)

Clause 66.03 of the planning scheme requires that an application for a planning permit in the BMO must be referred to the relevant fire authority.

## Definitions (clause 72)

Clause 72 sets out a definition for defendable space as:

- An area of land around a building where vegetation is modified and managed to reduce the effects of flame contact and radiant heat associated with bushfire.

## What complementary bushfire protection measures are there in the building system?

The Building Act 1993 includes requirements for bushfire protection as part of new development. These requirements apply to declared bushfire prone areas. Bushfire prone areas are a separate regulatory tool to those found in planning schemes.

A bushfire prone area covers:

- all areas of planning schemes within the BMO

- areas of moderate fuel loads such as unmanaged grasslands and smaller isolated patches of bushland

- areas with a moderate exposure to direct flame, radiant heat and ember attack.

In a bushfire prone area, a site assessment in accordance with AS 3959-2009 Construction of buildings in bushfire prone areas (AS 3959-2009) will generally be needed. This assessment should be undertaken by a suitably qualified person and must be completed to the satisfaction of the relevant building surveyor. The site assessment determines the appropriate Bushfire Attack Level (BAL) that a building must be constructed to. All buildings in a bushfire prone area must be constructed to at least a BAL 12.5, which provides for protection from ember attack.

A site assessment is also required for development subject to the requirements of the BMO, although where undertaken at the planning stage there are some key differences which need to be considered. These are outlined in more detail at clause 52.47 of planning schemes and in Practice note 65: Bushfire Management Overlay and bushfire protection: planning requirements.

Where a site assessment is done at the planning permit stage a further assessment is usually not required at the building permit stage.

## More information

More information about the new bushfire provisions introduced by Amendment VC83 is provided in:

Advisory note 33 – Bushfire protection: Community fire refuge and private bushfire shelter exemptions

Advisory note 39 – Bushfire protection: Vegetation exemptions

Practice note 64 – Local planning for bushfire protection

Practice note 65 – Bushfire Management Overlay and bushfire protection: planning requirements

More bushfire related information is available at www.dpcd.vic.gov.au/planning/bushfire

Published by the Victorian Government Department of Planning and Community Development Melbourne, November 2011.

© The State of Victoria, Department of Planning and Community Development 2011.

This publication is copyright. No part may be reproduced by any process except in accordance with the provisions of the Copyright Act 1968.

Authorised by the Victorian Government, 1 Spring Street, Melbourne.

This publication may be of assistance to you but the State of Victoria and its employees do not guarantee that the publication is without flaw of any kind or is wholly appropriate for your particular purposes and therefore disclaims all liability for any error, loss or other consequence which may arise from you relying on any information in this publication.

www.dpcd.vic.gov.au/planning