19/10/2026

VERY HIGH FLOOD RISK SCHEDULE [NUMBER] TO CLAUSE 44.09 FLOOD MANAGEMENT OVERLAY

Shown on the planning scheme map as FMO[number].

NAME OF SCHEDULE

Purpose

To identify land at very high flood risk (riverine, coastal and overland flows from stormwater) within the 1 per cent Annual Exceedance Probability (1 in 100 year) flood extent.

To limit the number of people at risk of very high flood risk.

19/10/2026

Flood management objectives to be achieved

Where no objectives are specified insert “None specified.”

or

Clause 44.09 specifies a schedule may contain flood management objectives to be achieved. Insert a maximum of 5 objectives.

19/10/2026

Statement of risk

Land in this overlay is subject to very high flood risk. Flooding is generally unsafe for people, vehicles and buildings.

Flood conditions can change rapidly with little warning, including sudden increases in water depth and speed, increasing forces on buildings and structures and the risk to human life.

Floodwater can:

    - Prevent people from remaining safely in place.

    - Cause people or vehicles to be swept from their location.

    - Rapidly cut access to or from the site.

    - Isolate properties and whole areas, including from emergency services.

    - Buildings and structures are vulnerable to structural damage and failure.

Flooding has the potential to result in severe impacts on:

    - Human life and public safety.

    - Buildings, property and other assets.

    - Emergency services and essential infrastructure.

    - Community infrastructure and services.

    - Public health through contaminated floodwater.

    - The natural environment.

    - Economic wellbeing and social cohesion of communities.

    - Cultural heritage values and places.

Clause 44.09 specifies a schedule may contain a statement of risk. Insert any additional statement of risk content.

19/10/2026

Requirements

### Use of land

### A permit is required to use land for:

    - Animal production (other than grazing animal production).

    - Art and craft centre.

    - Camping and caravan park.

    - Car park.

    - Crematorium.

    - Display home centre.

    - Dwelling.

    - Emergency services facility.

    - Energy generation facility.

    - Funeral parlour.

    - Industry.

    - Leisure and recreation (other than informal outdoor recreation).

    - Office.

    - Place of assembly.

    - Recreational boat facility.

    - Retail premises.

    - Small second dwelling.

    - Telecommunications facility.

    - Transport terminal.

    - Utility installation (other than minor utility installation).

    - Warehouse.

Any use listed in clause 62.01 if the corresponding requirement in clause 62.01 is not met.

Land must not be used for:

    - Accommodation (other than camping and caravan park, dwelling and small second dwelling).

    - Cemetery.

    - Community care accommodation.

    - Education centre.

    - Freeway service station.

    - Hospital.

    - Service station.

### Buildings and works

A permit is required to construct a building or construct or carry out works, including:

    - A fence.

    - Roadworks.

    - Bicycle pathways and trails.

    - Public toilet.

    - A domestic swimming pool or spa and associated mechanical and safety equipment if associated with one dwelling on a lot.

    - A domestic rainwater tank with a capacity of more than 4,500 litres.

    - A non-domestic rainwater tank with a capacity of more than 10,000 litres.

    - A pergola or verandah, including an open-sided pergola or verandah to a dwelling or small second dwelling with a finished floor level not more than 800mm above ground level and a maximum building height of 3 metres above ground level.

    - A deck, including a deck to a dwelling with a finished floor level not more than 800mm above ground level.

    - A disabled access ramp.

    - A permit is not required to construct a building or construct or carry out works for:

    - Roadworks, tramway, railway or bicycle paths and trails if any of the following apply:

  - The water flow path is not redirected or obstructed.

  - The roadworks, tramway, railway or bicycle paths and trails are constructed or carried out by or on behalf of the municipal council, public authority or the Head, Transport for Victoria, to the satisfaction of the relevant floodplain management authority.

    - Flood mitigation works, carried out by or on behalf of the floodplain management authority, public authority or the Head, Transport for Victoria.

    - An open-sided building associated with grazing animal production constructed on the same land or contiguous land in the same ownership.

    - The following works carried out to the satisfaction of the responsible authority:

  - The laying of underground sewerage, water and gas mains, oil pipelines, underground telephone lines and underground power lines provided they do not alter the topography of the land.

  - The erection of telephone or power lines provided they do not involve the construction of towers or poles.

    - Post and wire and post and rail fencing.

    - The following buildings and works if there is no significant change to existing surface levels or if the relevant floodplain management authority has agreed in writing that the flow path is not obstructed:

  - Landscaping.

  - Driveways.

  - Vehicle crossovers.

  - Footpaths and bicycle paths.

    - Buildings and works associated with agriculture, constructed or carried out to the satisfaction of the relevant floodplain management authority.

### Subdivision

A permit is required to subdivide land.

A permit must not be granted to create a vacant lot which is entirely within either the area affected by this schedule, or an area affected by this schedule and the high flood risk schedule to the Flood Management Overlay, unless any of the following apply:

    - A local floodplain development plan applies to the land.

    - The lot is to be transferred to an authority for a public purpose.

    - The relevant floodplain management authority has agreed in writing to the proposed subdivision.

### Local floodplain development plan

Name of plan incorporated into this scheme

An application must be consistent with the local floodplain development plan that applies to the land.

19/10/2026

Application requirements

An application must be accompanied by the following information as appropriate:

    - An existing conditions plan prepared by a licensed land surveyor showing boundaries and dimensions of the site and the layout and location of existing buildings and works with natural ground, existing finished floor and surface levels to Australian Height Datum.

    - A site development plan which includes:

  - The layout and location of proposed buildings and works including building entry and exit points and basement ramps.

  - Existing survey levels to Australian Height Datum.

  - The proposed finished building floor levels including in relation to building entry and exit points and basement ramps to Australian Height Datum.

  - The 1 per cent Annual Exceedance Probability (1 in 100 year) flood extent and the Nominal Flood Protection Level as determined by the relevant floodplain management authority.

  - The location of overland flow paths with the proposed buildings and works in place.

    - Elevations and cross-sectional drawings to include:

  - The proposed finished building floor levels to Australian Height Datum.

  - The proposed building entry and exit points and basement ramps and ground levels along access and egress routes within the property boundary to Australian Height Datum.

  - Flow paths for the passage of overland flows to Australian Height Datum.

  - The 1 per cent Annual Exceedance Probability (1 in 100 year) flood extent showing the proposed ground and finished floor level and the Nominal Flood Protection Level as determined by the relevant floodplain management authority.

### Flood risk report

If a local floodplain development plan for the area has not been incorporated into this planning scheme, an application must be accompanied by a flood risk report that addresses the following, where applicable:

    - The existing and proposed use and development of the land and the impact on flood risk.

    - Whether the proposed use or development could be located on flood-free land or land with a lesser flood risk not affected by this schedule.

    - The susceptibility of the development to flooding and flood damage.

    - The potential flood risk to human life, health and safety associated with the use or development. Factors to consider include:

  - The frequency, duration, extent, depth and velocity of flooding of the site and accessway.

  - The flood warning time available.

  - The danger to the occupants of the development, other floodplain residents and emergency personnel if the site or accessway is flooded.

  - The potential to isolate land not at risk of flooding from safe access within or outside this overlay.

    - The effect of the development on redirecting or obstructing floodwater, stormwater or drainage water and the effect of the development on reducing flood storage and increasing flood levels and flow velocities.

    - The effects of the development on environmental values such as natural ecosystems and habitats, stream stability, erosion and water quality.

19/10/2026

Decision guidelines

Before deciding on an application, in addition to the decision guidelines in Clause 65, the  responsible authority must consider, as appropriate:

    - Whether the proposal will result in an increase in the number of dwellings and people at risk of flooding.

    - Whether the proposed development achieves acceptable flood safety.

    - The likely or modelled extent of any impact of the development on floodwaters, including the nature and extent of impact on surrounding properties.

    - Whether any proposed strategies to minimise or mitigate risk of flood damage or safety will be effective over the life of the development.

    - Whether the proposed development maintains existing flood storage capacity and flow paths and conveyance.

    - Whether the proposal appropriately responds to the identified site-specific flood risk and the availability of a safe, practical and viable evacuation route.